Products

– Energy Control & Management

Smart Energy Meter

PC4713-W-E-TY/3W

# Description

PC4713-W-E-TY/3W helps you monitor the amount of electricity Consumed and Produced in your three-phase power system(included three phase three wire) by connecting the clamps on to the power cables. It is also compatible with split-phase and single-phase. It allows you to check real-time or historical energy data via mobile App.

# Main Features

• Tuya compliant. Support automation with other Tuya device by export and import of grid or other energy values

• Three-Phase(included 3P3W),Split-Phase and Single-Phase electricity system compatible

• Remotely monitor whole home Energy with three current transformers

• Ethernet helps you improve the Network stability

• Real-time Voltage, Current, PowerFactor, Frequency, ActivePower, ReactivePower, ApparentPower measurement

• Bidirectional measurement: Energy Usage / Solar Production

• Historical data could be displayed in Day, Week, Month and Year

# Main Specifications

| Connectivity |  |
| --- | --- |
| Wi-Fi | • 802.11 b/g/n @2.4GHz |
| BLE | • For Pairing |
| Ethernet | • 10/100 Mbps |
| Physical Specifications |  |
| Operating Voltage | • L-N：90~277 Vac 50/60Hz • L-L：156~480 Vac 50/60Hz |
| Supported Systems | • Single-Phase • Split-Phase • Three-Phase （included 3-Phase, 3-Wire） |
| Voltmeters Accuracy | • ±1% |
| Ammeters Accuracy | • 80A CT: ±1%（1-80A），±2%（0-1A） • 120A CT: ±1%（2-120A），±2%（0-2A） • 200A CT: ±1%（2-200A），±2%（0-2A） • 300A CT: ±1%（6-300A），±2%（2-6A），±5%（0-2A） • 500A CT: ±1%（6-500A），±2%（2-6A），±5%（0-2A） • 750A CT: ±1%（6-750A），±2%（2-6A），±5%（0-2A） |
| Clamp Inner Dimension | • 80A (default): Ø10mm • 120A(optional): Ø16mm • 200A(optional): Ø20mm • 300A(optional): Ø24mm • 500A(optional): Ø36mm • 750A(optional): Ø51mm |
| Reporting Cycle | • 15 seconds |
| Operating Environment | • Temperature: -20 ℃~+55 ℃ • Humidity: ≤ 90% non-condensing |
| Mount Type | • 35mm DIN rail mounting |
| Dimension | • 19(W) x 69(D) x 90(H)mm |
| Certification | •CE |